Replacing the Water Pump and Optispark

So, you say that you've lost high end rpm power and your car is doing some sputtering.  Does it sound like your cooking up a bag of pop-corn under the hood of your car?  Does it run and just randomly die?  Does heavy load cause loss of power?  Do you smell coolant under the hood around the front of your engine?  Is there a nice 8" or so splatter mark of dried coolant on your hood between the dust shields?  Do you hear missing or detonation a middle rpms?

If you answered yes to any of the above symptoms... there could be a chance your optispark and/or water pump have gone out on you.  Typically the average water pump seems to last around 75k miles.  Around this time, the bearing in the back of the pump that the propeller shaft rotates off of wears down.  Coolant then begins to leak out of the DESIGNED weep hole.  Yes, your water pump has been designed to leak when it fails.  And yes, you have guess correctly, it has been DESIGNED to leak right on top of your DISTRIBUTOR! (also known as your optispark).  Can the optispark go out on it's own without the water pump causing it to go out?  Yes.  Optisparks themselves seem to last around 100k miles, however the water pump usually destroys it before it even has a chance to show it's own typical failure.

The optispark is exactly as it sounds.  It's an optical spark device.  It simply uses an LED and a few small discs with 360 slots (for every degree in a circle) which are used for ignition timing.  Other than that, it can be considered like a normal distributor.  The problem with the design of this distributor is that it does NOT take kindly to moisture.  Moisture by itself causes the LED problems.  DO NOT WASH YOUR ENGINE!  Moisture will cause optispark failure.  Coolant is not just normal moisture, however.  Coolant is water mixed with corrosive chemicals which eat away at the seals and insides of the optispark.  When your water pump fails, it drips coolant on and into the optispark, and thus goes the story that I'd be willing to say nearly every LT1 owner has heard about or has had first hand experience with.  If you are hearing about this for the first time, beware, as you may be next!

No, nothing was done to your car to make it the devil like this.  It was DESIGNED this way.  You can thank the designers and engineers at GM for this.  Did they know this was going to be a problem, SURE THEY DID!  They knew that a weep hole dripping on the optispark would cause problems, but they also knew that it wouldn't happen until their liability for the car was out of the hands (ie the warranty was over).  So we, as the owners of the cars are left to pick up the pieces that GM left us, by replacing the water pump and optispark for usually a mere $450-1000, depending on where you get your parts.  The labor can easily make this a $1500 or so job!  Do yourself (and your wallet) a favor by doing the work yourself.  It's not hard, it just takes some time.